A

Software Package

  - Installation

You may need install some necessary driver before use Vecow HW monitor UI, please follow the steps to install driver.

Step: Run batch file “Win10_64.bat” as Administrator to install Vecow driver.

  - Uninstall

Step: Run batch file “Uninstall_64.bat” as Administrator to uninstall Vecow driver.

B

Start the HWMonitor UI program

Execute HWMUI.exe

HWMUI Function Panel

  - Main Page

Main Page for user choose subpage function.

- In the Subpage,can click the icon to return to MainPage.

  - Overview Page

Overview panel provides board information.

- Board Information

Indicate current motherboard revision, BIOS version and installed CPU information.

  - IO Access

IO Access control panel provides Isolated DIO/Non-Isolated GPIO Control and Status.

- Isolated Access

Bit login value

Bit Direct Status

DI Sink/Source

DI Read data

DO Sink/Source

Write DO data

ISO Mode

IO Port Select

Sink (NPN) / Source (PNP) radio button

Choose DI/DO mode.

Write button

Set DIO configuration and then set DO output state.

Read button

Set DIO configuration to get DI input state.

Output data (Hex)

DO output state by hexadecimal bitmask - on / off.

Range: 0x00~0xFF, 1: High; 0: Low.

Input data (Hex)

DI input state by hexadecimal bitmask - on / off.

Range: 0x00~0xFF, 1: High; 0: Low.

Example

Set Output data 0x00 LED all bright on sink mode.

Set Output data 0xFF LED all bright on source mode.

Loopback test (connect pin.1-11 ~ pin.10-20):

Set sink mode output and Get source mode input.

- Non-Isolated (GPIO) Access

GPIO Mode

IO Configuration (Hex)

GPIO In / Out, pin setting by hexadecimal bitmask.

Range: 0x0000~0xFFFF, 1: Out; 0: In.

IO Status (Hex)

GPIO output/input state by hexadecimal bitmask - on / off.

Range: 0x0000~0xFFFF, 1: High; 0: Low.

Update button

Update GPIO configuration and GPIO input state.

Example

Loopback test (connect pin.1-11 ~ pin.10-20):

Set IO data and Get IO data.

  - POE Device Configuration

- Mode Switch

User config Auto/Manual Mode

- Power switch

User settings on/off

  - Thermal Control

- Detect CPU/GPU DTS Temperature value

- CPU-TJMAX temperature

The maximum temperature limit by CPU design.

- Fan Fail Detect

If current RPM smaller than protected RPM over 1 minute, then machine will shut down system in 1 second.

- Start button : Start detected FAN

- Stop button : Stop detected FAN

- Fan Mode Selection

- Manual Mode

Manually set the fan speed(0~100%).

- Smart Mode

    - User can choose presets or custom.

    - Default mode

        - Fan start run temperature:45 degrees Celsius.

        - Start speed: 60%

        - Temperature at full fan speed : 90 degrees Celsius.

    - User mode

User can define parameters.

(After the program is restarted,the parameters need to be reset.)

  - Uart

- User can configure COM Port mode in RS232,RS422,RS485,loopback.

- When the RS422/RS485 mode is selected,the terminal resistance can be enabled or disabled.

  - USB ON/OFF

When using Vecow USB expansion card(UE-1004/UE-1008),set on/off for single port.

  - Watchdog

Test the watchdog function.

  - SMBUS

- SMBUS panel provides BUS data accessed on SMBUS.

- BUS data function support byte transmission.

- Slave Address

The SMBUS slave device address is expected in 8-bit hex format with the 7-bit address in the upper seven bit and the lowest bit set as read(1) / write(0) bit accordingly.

- Register/Command

The SMBUS command enabled access to SMBUS devices.

- Data

Support transfer type: Read/Write Command Byte.

- SMBUS control PoE

  - PoE slave address

Range: 0x40~0x5E (16 address).

  - Set 4-port PoE Configuration for auto mode

Register 0x12, Write 0xFF.

Register 0x13, Write 0xF0.

Register 0x14, Write 0xFF.

  - Set 4-port PoE Configuration for manual mode and trun on/off PoE port:

Register 0x12, Write 0x55.

Register 0x13, Write 0x00.

Register 0x14, Write 0x00.

Register 0x19, Write 0x0F/0xF0.

  - Ignition

Vecow specific product models offer electronic ignition power control feature for in-vehicle applications.

On/Off delay time can be set by sofeware or hardware(via ignition jumper).

- Ignition mode

Auto Detect Hardware/Sofeware ignition mode.

- Power mode Switch

  - Normal :System power on by power button.

  - Ignition: System power on by Ignition pin.

- Delay On Timer(Seconds)

The delay time after user trigger ignition on signal(Seconds).

- Delay Off Timer(Seconds)

The delay time after user trigger ignition off signal(Seconds).

- Force Shutdown Timer(Minutes)

Used to force cut off system power when OS unable gracefully shutdown system successfully.

- Voltage Guard

Voltage Guard enable or disable, only effect on lgnition mode.

- Voltage Guard lower limit value

Voltage Guard lower limit value setting. Range:9v – 40v.

- Voltage Guard higher limit value

Voltage Guard Higher limit value setting. Range:15v – 55v.